1. What is likely the sequence of events the user performed to produce such a packet trace?

- User wants to visit www.nyansa.com

- For which it needs an IP address, so a DHCP Request is sent out from client

- DHCP server allocates an IP ( 192.168.1.80) for the client

- Client sends the request to DNS server to resolve IP for www.nyansa.com

- DNS server resolves the IP as 54.231.236.19

- Client sends the HTTP request over TCP/IP to the server at 54.231.236.19 and continues to browse the content of www.nyansa.com

2. What is the IP of the client device and what is the IP address of the DHCP server?

Client IP address: 192.168.1.80 (after DHCP allocation)

DHCP Server: 192.168.1.1

3. What is response time of the DHCP server?

1.157273 seconds (Delta between the time stamps of first DHCP Request and DCHP ACK)

4. What IP did nyansa.com resolve to?

54.231.236.19

Derived from DNS Response Packet:

5	3.880846	192.168.1.1	192.168.1.80	DNS	114	Standard query response 0x2ac7 A www.nyansa.com CNAME nyansa.com A 54.231.236.19

5. What is the make of client device and what OS version is it running?

Macintosh; Mac OS X 10_9_5

Derived from user-agent field of HTTP:

user-agent: Mozilla/5.0 (Macintosh; Intel Mac OS X 10_9_5) AppleWebKit/537.36 (KHTML, like Gecko) Chrome/42.0.2311.152 Safari/537.36

6. What is the MAC address of the client's gateway?

ac:22:0b:33:09:e0

Derived from L2 header:

Ethernet II, Src: Apple_7c:87:7b (20:c9:d0:7c:87:7b), Dst: ASUSTekC_33:09:e0 (ac:22:0b:33:09:e0)

7. How long did it take to complete the first 3-way TCP handshake to nyansa.com?

0.013255

8. What type of browser does the client device have?

Chrome/42.0.2311.152

Derived from:

user-agent: Mozilla/5.0 (Macintosh; Intel Mac OS X 10_9_5) AppleWebKit/537.36 (KHTML, like Gecko) Chrome/42.0.2311.152 Safari/537.36

9.  What is the total number of TCP sessions?

6

10. What is the HTTP response time for the first HTTP request to nyansa.com?

0.386230

11. What VLAN is the client on?

There are no tagged packet capture to determine this.

12. What is total number of HTTP requests?

35